# Project 2: VLAN and Inter-VLAN Routing with FortiGate

Project Overview

In this project, our team configured a complete set of firewall policies on a FortiGate device to manage traffic between multiple VLANs and secure access to internal and external resources. We designed the network with full segmentation, ensuring that each VLAN has a specific purpose and predefined privileges. Through these policies, we achieved secure traffic control, strong segmentation, and structured access across the entire environment.

1. Network Topology Overview

The network is centered around a FortiGate firewall acting as the gateway and security enforcer.

- Port1 (LAN): Connects to a Cisco Switch, which trucking multiple VLANs to end-user VPCs (HR, IT, Sales, etc.).

- Port2 (WAN): Connects to the "Cloud1" (Internet) via DHCP.

- Port3 (Router Link): Connects to a Router (R1) with IP 192.168.110.1, used for routing and loopback managemen

2. VLAN Configuration & Segmentation (Port1)

The internal network is segmented into 8 VLANs to ensure security and efficient management. All VLANs are configured on Port1 with the subnet mask /24 (255.255.255.0)

| VLAN ID | Name | IP Address | Purpose |
| --- | --- | --- | --- |
| 10 | Management | 192.168.10.1 | Administrative access and network management. |
| 20 | HR | 192.168.20.1 | Sensitive employee records and payroll. |
| 30 | IT | 192.168.30.1 | System maintenance and troubleshooting. |
| 40 | Finance | 192.168.40.1 | Accounting and financial transaction data. |
| 50 | Sales | 192.168.50.1 | CRM systems and client management tools. |
| 60 | Marketing | 192.168.60.1 | Internet-based operations and digital campaigns. |
| 70 | Guests | 192.168.70.1 | Isolated internet access for visitors. |
| 80 | Servers | 192.168.100.1 | Hosts internal servers and critical applications. |

3. Address Groups

To simplify policy management, VLANs are grouped into logical objects.

- Access Internet : Includes VLAN60 (Marketing) and VLAN70 (Guests). These groups are allowed to browse the web.

- Access Server: Includes VLAN20 (HR), VLAN40 (Finance), and VLAN50 (Sales). These departments require access to business servers on VLAN80.

- Network Maintenance: Includes multiple VLANs. This is used by IT & Heads to monitor and troubleshoot the network support, monitoring, and administrative tasks.

- Protected VLANs: This includes all internal VLANs. This group is used to define what the guest network is explicitly forbidden from accessing, as well as what it is isolated from.

- Admins: VLAN10 and VLAN30 for IT and Heads to manage and monitor

4. Firewall Policies Analysis

The configuration file defines several firewall policies (rules) that control traffic flow based on the documentation provided.

Policy 2: Access Server

- Traffic: From HR, Finance, Sales (Access Server Group) to Servers (VLAN80).

- Action: ACCEPT.

- Service: ICMP, HTTPS, SMB, FTP.

- Purpose: Allows employees to access internal file servers and applications needed for their work.

Policy 3: Internal VLANs Protection (Guest Isolation)

- Traffic: From Guests (VLAN70) to Protected VLANs (Internal Network).

- Action: DENY.

- Purpose: Blocks guest users from accessing any internal company resources to maintain security.

Policy 4: Updates

- Traffic: From Servers (VLAN80) to WAN (Port2).

- Action: ACCEPT (with NAT enabled).

- Schedule: "Time_For_Update" (Saturdays 19:00 - 22:00).

- Purpose: Allows servers to reach the internet for OS updates and time synchronization (NTP) only during a specific maintenance window.

Policy 5: Internet Access

- Traffic: From Marketing & Guests (Access Internet Group) to WAN (Port2).

- Action: ACCEPT (with NAT enabled).

- Purpose: Provides standard internet browsing capabilities.

Policy 6: Network Maintenance

- Traffic: From IT & Management (Admins) to  All VLANs (Network Maintenance Group).

- Action: ACCEPT.

- Purpose: Give IT and Heads full access to all network segments for administration and troubleshooting.

Router & Loopback Policies (Policies 7, 8, 9, 10 in Config)

These policies manage traffic moving through Port3 (connected to Router R1) and the Loopback Interface.

- Policy 7 (Router To Server): Allows traffic from the Router (R1) to access the Internal Servers (VLAN80).

- Policy 9 (Manage Other Branch): Allows IT/Heads (VLAN 10, 30) to manage the remote router and Loopback interface using PING, SNMP, and SSH.

- Loopback Security: The documentation notes a policy to Deny Guests to Loopback. In your config, this is handled effectively because there is no "Allow" policy for guests to reach Port3, so the Implicit Deny will block it automatically.

5. Routing Configuration

- Default Route: All internet traffic points to the gateway on port2 (WAN).

- Static Route to Loopback: A static route is configured to reach the Loopback IP 100.100.100.100 via the Router 192.168.110.2 connected to port3

Key Infrastructure Components

- Central Gateway: The FortiGate firewall acts as the core security device, managing traffic between the internal Local Area Network (LAN), the Wide Area Network (WAN), and a routed management link.

- VLAN Segmentation (Port1): The internal network is divided into 8 distinct VLANs to isolate traffic based on department function.

  - Operational VLANs: HR (VLAN20), Finance (VLAN40), and Sales (VLAN50) handle sensitive business operations.

  - Support VLANs: Management (VLAN10) and IT (VLAN30) are dedicated to administration and system maintenance.

  - Restricted VLANs: Guests (VLAN70) are provided internet access but are strictly blocked from communicating with internal networks.

  - Infrastructure: Servers (VLAN80) host critical applications and databases.

Security & Policy Design

The firewall policies are designed around the principle of Least Privilege, granting users only the access necessary for their roles:

- Logical Grouping: VLANs are organized into groups such as "Access Server" (HR, Finance, Sales) and "Access Internet" (Guests, Marketing) to simplify policy management.

- Traffic Control:

  - Server Access: Specific policies allow business units to access internal servers via secure protocols like HTTPS and SMB.

  - Guest Isolation: A "Protected VLANs" group is used to explicitly DENY guest traffic from reaching sensitive internal networks.

  - Maintenance: A dedicated policy allows the IT department to access all VLANs for monitoring and troubleshooting.

Management & Reliability

The configuration includes a Loopback Interface (Loopback_port3). This is a virtual interface designed to provide a stable IP address for administrative tasks (like SSH and SNMP monitoring) that remains accessible even if physical interfaces change or flap, ensuring consistent network management.